Software Setting Instruction of QRcode Reader

1.Software interface

2. Definition and explanation of interface

2．1 Wiegand Output Parameters

2.1.1 Wiegand Output PulseWidth and PulsePeriod

The output of Wiegand is a negative pulse signal. Usually, the Wiegand signal is high level. When one bit of binary data is output, the signal becomes low level, lasts for a period of time, and then becomes high level. The duration of signal at low level is called pulse width, and the shortest time from low level to high level to the next low level is called pulse period.

The Wiegand pulse width is more wide, the lower the speed requirement for the access controller decoding  will be.It is suggested that the wiegand pulse width should be 200-500us

2.1.2 Wiegend output formats

Common Wiegend output formats are Wiegand26, Wiegand32 and Wiegand34. There are also Wiegand 42, Wiegand 44 and Wiegand 66 formats, which can be customized for special special project.

The Wiegand protocol is to output binary in the form of negative pulse. When binary 0 is output, Wiegand signal D0 outputs negative pulse. When binary 1 is output, wiegand signal D1 outputs negative pulse.

The Wiegand-26 outputs 3 bytes of data, 1 byte = 8 bits, 3 bytes = 24 bits. The first bit of Wiegand-26 is even check bit, and the last bit is odd check bit. So the total number of bits is 26;

The Wiegand-34 outputs 4 bytes of data, 1 byte = 8 bits, 4 bytes = 32 bits. The first bit of wiegand-34 is even check bit, and the last bit is odd check bit. So the total number of bits is 34;

The Wiegand-32 outputs 4 bytes of data, 1 byte = 8 bits, 4 bytes = 32 bits. The Wiegand-32 has no parity bit

2.1.3 Output Byte Order

“Data Output Byte Order” and ” Card Output Byte Order”， It refers to the byte output by Wiegand, whether the high byte is output first or the low byte is output first.

“Data Output Byte Order”， It refers to Qrcode data or serial port input data, intercepting a section of data for wiegand output。

“Card Output Byte Order”，It refers to the order of card serial number output.

The Wiegand-26 outputs 3 bytes of data. The output order of these 3 bytes is "byte 1 -> byte 2 -> byte 3", which is called forward output. The output order of these 3 bytes is "byte 3 -> byte 2 -> byte 1", which is called reverse output; Similarly, The wiegand-34 outputs 4 bytes of data. The output order of these 4 bytes is "byte 1 -> byte 2 -> byte 3 -> byte 4", which is called forward output. The output order of these 4 bytes is "byte 4 -> byte 3 -> byte 2 -> byte 1", which is called reverse output

2.1.4 Input data format of serial port (QR code )

“Input data format of serial port”， It refers to the data input through the serial port, or the data obtained by scanning the Qrcode or Barcode. These data are formatted.

cStringType.AddString("Numeric string");

cStringType.AddString("Hexadecimal string");

cStringType.AddString("Hexadecimal data");

cStringType.AddString("ASCII string");

At present, these data formats are defined into four categories：

1) Numeric string,

2) Hexadecimal string

3) Hexadecimal data

4) ASCII string

- “Numeric string”, the input data is a 0-9 numeric string. For example: "12345678909876543210"

- “Hexadecimal string”, The input data is a hexadecimal string. For example:"A0B1C2D3E4F510213"

- “Hexadecimal data”, The input data is hexadecimal data. For example: 0xA0B1C2D3

- “ASCII string”, The input data is a string of numbers and other characters,For example:"1269abcACEGH"

2.1.5  Get the positon of the input character

(1)“Get from start”

the output Wiegand data is obtained from the beginning. For example, if the numeric string is "12345678909876543210" and Wiegand-26 outputs it, the valid string obtained is "12345678"

the output Wiegand data is obtained from the beginning, then the start position value is equal to 0, the end position value is equal to 0.

(2)“Get from end”

The output Wiegand data is obtained the characters at the end of the string. For example, if the numeric string is "123456789098765443210" and Wiegand-26 outputs, the valid string obtained is "76543210"

The output Wiegand data is obtained the characters at the end of the string, then the start position value is equal to 1, the end position value is equal to 0

(3)“Start and end positions”

The output Wiegand data is obtained by specifying the start and end positions. For example, if the number string is "12345678909876543210" and Wiegand-26 outputs, the start position is 0 and the end position is 7, then the valid string obtained is "12345678"; if Wiegand-26 outputs, the start position is 3 and the end position is 10, then the valid string obtained is "45678909"

2．2 Read Mifare Card Sector Data Parameters

2.2.1 Enable Read Sector of Card

“Enable Read Sector of Card”, It refers to reading the contents of the sector in the Mifare card

2.2.2 Enable Card_Wiegand Output

“Enable Card_Wiegand Output”, It refers to swiping Mifare card, which will enable Wiegand output.

2.2.3Sector No.

Mifare 1 card has 16 sectors, sector No. 0-15, one sector has four block, 16 bytes per block, 64 bytes in total, and 48 bytes are user data area

If check ” Enable Read Sector of Card”, RS232 will output 48 bytes of data for the whole sector。

2.2.4  Sector StartPosition

If check ” Enable Read Sector of Card”function，and check ” Enable Card_Wiegand Output”, Then we need to get 3-4 bytes  in the sector for wiegand output。” Sector Start Position”, It refers to the location of the sector  to start get data.

2.2.5  Sector Secret KeyA

“Sector Secret KeyA”, It means that each sector of each Mifare card has a key. If you need to read and write the contents of a sector of the card, you need to check the key of the sector. So you need to Set the key to the device in advance. When reading and writing the sector of the Mifare card, you need to read and write the Mifare card with the key

3. User instructions

There are two kinds of input data for QRcode reader, one is QRcode data or serial port input data, the other is reading Mifare card number or sector data in Mifare card. And there are two kinds of output data, one is RS232 or RS485 output data, the other is Wiegand output (Wiegand26, Wiegand32, Wiegand34)

Wiegand data is relatively small data. Wiegand 26 has only 3 bytes,and Wiegand34 data has only 4 bytes. The data input through serial port is relatively long. If several bytes are intercepted, these parameters need to be set. In the same way, if the data read from the sector in the card is relatively long, if a few bytes are intercepted for Wiegand output, It also needs to set parameters to make it flexible to adapt to the use of field controller.

If the device firmware needs to be upgraded, it can be upgraded with RS232 serial port, and it can be upgraded with special download software

Note：

1）Upgrading will restore the parameters to the default settings, so you need to read the parameters before upgrading, and set the parameters again after upgrading.

2）The serial port baud rate is currently fixed at 9600, which can be adjusted by firmware upgrade if necessary

3.Packet communication protocol for RS232

| No | Name | Bytes | Symbol | Explain |
| --- | --- | --- | --- | --- |
| 1 | start | 1 | START | Start character, fixed:0x02 |
| 2 | Data length | 2 | LENGTH | The data length is equal to all the data lengths except the start character, the ending character, and the data length itself. data length = all data length -4 |
| 3 | Device address | 6 | ADDRESS | The MAC of Device |
| 4 | Device Type | 1 | TYPE | Identification of different types of device.accessAccess control equipment is fixed to ‘0x11’, The device type is shown in table 1-3 |
| 5 | command | 2 | COMMAND | Different functional communication commands are different |
| 6 | Return state | 1 | STATUS | returns the status value, the sender fills the 0xFF with this parameter, and the device returns the status value as table 1-2 |
| 7 | Command data | N | DATA | If the communication command does not require data, the field is empty. |
| 8 | Verifying word | 1 | CHECK | XOR values for all bytes from the data length (including the data length) to before the checkword (excluding the checkword itself)。 |
| 9 | End character | 1 | END | The end byte is fixed to 0x03 |

3.1 Scan QRcode and output data

020037000000000001A155060068747470733A2F2F752E7765636861742E636F6D2F4D446A75365F574675754F5653634A59364A495F5F76348103

Data length:0037H

Command:5506H

Command data:

68747470733A2F2F752E7765636861742E636F6D2F4D446A75365F574675754F5653634A59364A495F5F7634

Character string: https://u.wechat.com/MDju6_WFuuOVScJY6JI__v4

3.2 Swipe Mifare1 card and output card series number

02000F000000000001A1510000077C18BD2003

02000F000000000001A1510000075F991D2203

Data length:0FH

Command:5100H

Command data: 077C18BD(card series)

4. Hardware wiring instructions

Black：GND (Power GND)

Red： 12V (12V DC power supply)

Green：Wiegand D0 output

White：Wiegand D1 output

Blue： RS232 TXD  Output  or  RS485A

Yellow：RS232 RXD Input or  RS485B

Brown：GND (Power GND)